- Explain intuitively why the effective index of a strip waveguide typically decreases with wavelength.

- Explain intuitively why the effective index of a strip waveguide typically increases as the waveguide width is increased.

- Explain intuitively why the group index of a strip waveguide typically decreases as the waveguide width is increased (e.g. from 400 nm to 600 nm).

- Design a waveguide with zero group velocity dispersion at 1550 nm.

- Design a grating coupler for TM 1310 nm (straight fingers), for an incident angle

of 10◦.

- Design a grating coupler for TM 1310 nm (focusing).

- Design and optimize an edge coupler for TE 1310 nm operation, coupled to an ideal lens assembly.

- Determine the thermal tuning efficiency for a thermal phase shifter where the heater is embedded in the waveguide via a N++/N/N++ region. Assume the waveguide is a rib waveguide with a 10 μm slab width, 90 nm slab thickness, with a ridge that is 0.5 μm width and 220 nm thick.

- Consider a ring resonator with 220 nm × 500 nm strip waveguides that is uniformly heated. Determine an expression for the wavelength shift versus temperature, i.e. dλ/dT, at 1550 nm. How does this vary with the radius of the ring resonator?